RDZHT-4N1X  Transformer Multifunctional Tester

In the power system, transformers serve as core equipment, and their long-term reliable operation is crucial to the stability and reliability of the entire system. Efficiency classification, DC resistance, voltage ratio group, and insulation resistance tests are mandatory items for transformer power-off maintenance. Traditional testing requires separately configuring four independent instruments, which are inconvenient to carry and necessitate frequent disassembly and reconnection of specialized test leads, consuming significant manpower, material resources, and labor hours. Our company has innovatively developed a four-in-one distribution transformer testing system, integrating four testing functions—DC resistance, voltage ratio, insulation resistance, and efficiency classification—into a single unit. Powered by high-capacity lithium batteries, this system enables multiple tests with a single wiring setup, eliminating the need for frequent test lead replacements, thereby significantly enhancing testing efficiency and convenience.

- Product features

- The wiring is convenient and efficient, allowing both DC resistance and turns ratio tests to be completed with a single connection.

- The direct resistance testing function is powerful. Transformers with Yn, Y, and △ windings can adopt three-phase automatic testing method and calculate the three-phase unbalance rate. The maximum output current can reach 20A, which can meet the testing requirements of extremely low resistance of low-voltage coils in distribution network transformers.

- The turn ratio test has the function of testing three-phase, single-phase, and Z-type transformers, and supports blind testing as well as PT/CT turn ratio and polarity testing.

- The energy efficiency level testing function covers capacity, load, no-load, and zero sequence impedance testing.

- The maximum output short-circuit current for insulation resistance testing is 5mA, and it can be adjusted within the range of 1mA to 5mA. Equipped with IR, DAR, and PI testing functions.

- Equipped with an embedded Linux operating system, it has high security, smooth operation, and convenient operation.

- Adopting A7+M4 dual core architecture, balancing real-time measurement and smooth operation.

- Equipped with an industrial grade 7-inch large screen, the display style is fresh and simple, supporting touch operation and rotating mouse operation, providing a comfortable and convenient human-machine interaction experience.

- Equipped with a large capacity lithium battery and a built-in charger, the power supply is convenient and reliable

- Product specifications and technical parameters

| Item |  | Description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DC resistance test | Test current | 20A, 10A, 5A, 1A, 0.1A, ≤10mA |  |  |  |
|  | Range | 0.5mΩ ~ 100mΩ @20A ; 1.0mΩ ~ 300mΩ @10A ; 10mΩ ~ 1.0Ω @5A 200mΩ ~ 6.0Ω @1A ; 2.0Ω ~ 60Ω @0.1A ; 30Ω ~ 50kΩ @≤10mA |  |  |  |
|  | Accuracy | ±(0.2%rdg+2dgt) |  |  |  |
| TTR test | Range | 0.9 ~ 5000 |  |  |  |
|  | Accuracy | ±(0.1%rdg+2dgt) @0.9 ~ 500 ; ±(0.2%rdg+2dgt) @500.01 ~3000 ; ±(0.3%rdg+2dgt) @3000.1 ~ 10000 |  |  |  |
|  | Resolution | 0.0001 @0.9 ~ 0.9999 0.001 @10 ~ 99.999 0.01 @100 ~ 999.99 0.1 @≥1000 |  |  |  |
| Transformer capacity, no-load and load testing | Range | Voltage: AC 50V ~ 500V; Current: AC 0.5A ~ 50A; Capacity: 30kVA ~ 65000kVA |  |  |  |
|  | Accuracy | Voltage & Current: ±(0.2%rdg+2dgt)；Power(0.2≤cosφ≤1): ±(1.0%rdg+2dgt)；Capacity: ±(10%rdg+2dgt) |  |  |  |
| Insulation resistance test | Output voltage | 500V, 1000V, 2500V, 5000V |  |  |  |
|  | Short circuit current | Adjustable 1~ 5mA, step 1mA |  |  |  |
|  | Current measurement | Range: 5mA; Accuracy: ±(5%rdg+0.2nA) |  |  |  |
|  | Capacitance measurement | Range: 20μF; Accuracy: ±(15%rdg+0.03nF) |  |  |  |
|  | Capacity discharge rate | From 10000V to 10V, 0.5s/μF |  |  |  |
|  | Voltage Accuracy | 500V | 1000V | 2500V | 5000V |
|  | 5% | 100kΩ~10GΩ | 100kΩ~20GΩ | 100kΩ~100GΩ | 100kΩ~100GΩ |
|  | 20% | 10GΩ~100GΩ | 20GΩ~200GΩ | 100GΩ~1TΩ | 100GΩ~1TΩ |
| Display |  | 7-inch LCD screen |  |  |  |
| Power supply |  | Rechargeable lithium batteries; Charger input 100-240VAC, 50Hz/60Hz. |  |  |  |
| Operating conditions |  | Temperature -10 ~ 50℃; Relative humidity ≤90%, no condensation. |  |  |  |